1. Block

- Factory GM OEM blocks allowed.

| Part Number | Mfg | Material | Displacement | Max Bore |
| --- | --- | --- | --- | --- |
| GMP12673475 | GM Performance | Iron | 6.0 L | 4.030” |

- Block lightening of any kind is not allowed. All mounts must remain. This includes fuel

pump, motor mounts, and all other stock mounts.

- Oversized and roller cam bearings are not allowed.

2. Crankshaft

- Stroke length must be  3.622” +/-0.005”.

- No lightening, counterweight knife edging, or counterweight polishing is allowed. Balancing is allowed.

3. Connecting Rods

- Connecting rod length must be 6.098-6.125.

- Connecting rods and bolts must be magnetic steel.

4. Pistons/Rings

- Any forged or cast aluminum piston is allowed.

- Engines must not exceed 12.3:1 compression ratio with the heads and gasket installed.

5. Valvetrain

  - Only hydraulic lifters with .700" OEM diameter roller wheel and .842 diameter are allowed.

  - Push rods must be a stock, or stock type replacement, with a 5/16” diameter.

  - Roller tip rockers are not allowed.

  - Rockers must be 1.7:1 ratio.

  - Maximum lift at the valve is .600”.

6. Cylinder Heads

- Only the following unmodified cylinder heads per manufacture’s specification are allowed.

| Part Number | Mfg | Intake Valve Dia. | Exhaust Valve Dia. | Chamber CC | Intake Port Volume CC | Casting # |
| --- | --- | --- | --- | --- | --- | --- |
| 88958758 | GM Performance | 2.165 | 1.59 | 70 | 276 | 821, 823 |

- Angle milling, material removal or addition in any area of head is not allowed.

- No porting, grinding or polishing of any kind is allowed.

- The seats and guides may be replaced.

  - Valve springs must be magnetic steel.

  - Valve spring retainer may be steel or titanium.

7. Intake: Naturally aspirated induction only.  Turbo charging or super charging is not allowed.

7a. GM Performance intake manifold Part Number 25534401

7b. Holley Part Number 300-131

7c. Edelbrock Victor Jr. Part Number 28457

7d. Edelbrock Super Victor Part Number 28265

- No modifications are allowed.

- No porting, polishing, or grinding of any kind is allowed.

8. Fuel Delivery System

4 Barrel Carburetor: Spec Holley 750 cfm. The only allowed changes are jetting,

power valves, and air bleeds. No other modifications are allowed. Carburetor

will be subject to 1.375” venturi Go / no-Go gauge checking as part of tech.

9. Fuel

- Racing methanol only. Gasoline is not allowed.

- Nitro, nitrous oxide, or any other oxidizer, with the exception of methanol, is not allowed.

- No fuel additives, other than upper cylinder lubricants, are allowed. Fuel is subject to inspection at any time.

10. Oiling System

- Wet sump oil pan reservoir system is required.

- OEM type pump, mechanically driven from crankshaft snout, is required.

11. Ignition

Electronic Spark-only control of OE style coils on plug

  - An MSD #6010, MSD #6014, MSD #6014CT or MSD #60143 ignition controller is required.

  - Only OE or physically compatible OE replacement coil packs are allowed.

  - Only nominal 12V lithium-ion, Iron-phosphate, or NMC batteries are allowed. The battery must be securely hard mounted and shielded in the same area designated for ballast mounting.  Bus voltage must read less than 14.8 V at any time.

- On board starters are not allowed.

- No electronic traction control devices are allowed.

- The ignition firing order must remain stock.

12. Exhaust:

- Headers with maximum 1-3/4 tube diameter per cylinder, up to collector are required.

- Schoenfeld 14272735‑78 Muffler is the only muffler allowed.

- Tri Y headers are not allowed.

13. Water Pump & Radiator

- Any mechanically driven water pump is allowed.

- Electric water pumps are not allowed.

14. Fuel Pump

- A mechanical, belt driven, or cam driven fuel pump is allowed.

- Electric fuel pumps are not allowed.

15. Exotic Materials

- No Titanium, Inconel, Ceramics, DLC, or Nikasil components are allowed.